| # REPBULIC OF RWANDA MINISTRY OF HEALTH …………. PROVINCE …………. DISTRICT …………. HOSPITAL P.O Box …………….. # Email: ……………… | Policy/Procedure Title: Sharps and Needles Disposal |  |  |
| --- | --- | --- | --- |
|  | Policy code/ Number: ES3-16 | # Effective Date: August 2018 | # Revision date: July 2020 |
|  | Department: Clinical Department | Applies to: All clinical staff and Cleaners |  |

|  | Position | Names | Date& Signature |
| --- | --- | --- | --- |
| Responsible | Environmental Health Officer |  |  |
| Supervisor | Director of Medical and Allied Health Sciences Services |  |  |
| Approval | Director General |  |  |

# Purpose: To ensure that used sharps and needles are safely disposed in order to prevent risks to hospital staff, patients, visitors, and the environment.

# Policy Statements:

- The hospital shall ensure that sharps and needles are properly disposed in a timely manner.

- The hospital shall ensure that staff, patients, and housekeepers are trained on the proper handling and disposal of sharps and needles.

- The hospital shall ensure needles and sharps are incinerated in accordance with the developed plan.

Equipment and Forms:

- Sharps containers

- PPEs (Personal Protective Equipment)

- Trolley

Procedure:

- Avail sharp containers in relevant areas, which are labelled “infectious” and “non-infectious.”

- Containers must be located at accessible and secured point prior to use ensure proper segregation to minimize risk of injury.

- Do not keep sharp containers on the floor, they must be kept on the trolley.

- Dispose needles/sharps into container correctly without recapping, bend, break, or other manipulation.

- Discard sharps into the container at the point of use.

- Take the sharps container with you when completing a procedure requiring the use of disposable sharps.

- The health care provider locks down the lid when a sharps container is 3/4 full and replaces it with the new one.

- The cleaner removes and disposes of the locked containers.

- Transportation is done by trained staff with recommended PPEs (heavy duty gloves and protective clothing).

- Needles/sharps containers are transported on trolley to temporary storage and secured area located at incinerator building.

- When the incinerator is not in good working condition, needles and sharps containers are stored and transported for incineration in the nearest hospital as soon as possible.

- The cleaner removes the PPEs and washes hands.

- Standard precautions must be posted in each clinical area for the management of sharps and needles.

- To ensure sustainable management of sharps and needles, MoU (Memorandum of Understanding) must be signed with other nearest hospitals for intervention during breakdown of incinerator.

- After the incineration of sharps and needles, the ash produced shall be disposed in a very deep, cemented, and secured pit.

References:

- World Health Organization. World Alliance for Patient Safety: Forward Programme. World Health Organization; 2005. Control

- Health Information and Quality Authority. National Hygiene Services Quality Review (2008) Standards and Criteria. Ireland: Health Information and Quality Authority; (2008).

Appendix

SCHEDULE OF ELIMINATION OF SHARPS AND NEEDLES

| No | HOUR | FROM | TO |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |

SCHEDULE OF INCINERATIONOF SHARPS AND NEEDLES

| No | Days | Date | Quantity in Kg (TBD) | Remained quantity | Comment |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |

Done by: ……………………………                           Approved by: ……………

Incinerator Operator                                                    Environmental Health Officer